S88618

Maintenance manuals

LENOVO

Catalogue

1. Teardown guidance	3

2. The main board layout is introduced	10

2.1 The mainboard positive figure	10

2.2 Main board on the back of the figure	11

2.3 Motherboard, figure a PDF document	11

2.4 The main board schematic PDF document	11

3.Failure analysis flow chart	12

3.1 Don't start up	12

3.2 Touch screen failure	13

3.3 charge fault	14

3.4 The receiver malfunction	15

3.5 The horn fault	16

3.6 LCD display failure	17

3.7 The key fault	18

3.8 Mobile communication failures	19

3.9 MIC fault	20

3.10 Camera malfunction	21

3.11 Vibration fault	23

3.12 SIM card not to identify	24

3.13 SD card can't identify	25

3.14 FM fault	26

3.15 Bluetooth/WLAN fault	27

3.16 Fingerprint fault	28

Teardown guidance

a. Disassemble battery cover

Turn on the heating platform, and regulate temperature controller, set the target temperature to 80 degrees；Remove the SIM tray  by SIM needle；After the heating platform is heated to 80 degrees, put the phone screen upwards on the heating platform horizontally.

b. Disassemble battery cover

After the phone is heated for 3-4 min , take the phone with the heat insulation gloves；

Put the phone screen upwards on the battery cover disassembly fixture immediately；Open the disassemble equipment，To separate the battery cover.

c. Disassemble battery cover

Take the phone out of fixture，To Separate the battery cover with a plectrum；After the battery cover separated completely, Use tweezers to remove the fingerprint FPC bracket；Remove the fingerprint FPC from the main board.

d. Disassemble the rear housing and main antenna bracket

After separating the battery cover， cleaning the adhesive remains from the battery cover  and the front housing module；Screw off the screws on the rear housing by screw driver；Screw off the screws on main antenna bracket by screw driver；Separate the rear housing and main antenna bracket from the front housing module。

e.  Disassemble the main board

Remove the battery FPC from the main board；Remove the LCD FPC and main FPC from the main board；Remove the side key FPC and p-sensor FPC from the main board；Remove the coaxial cable connector from the main board

f. Disassemble the main board

Tear off the Copper foil of the front camera；Separate the rear camera and the front camera from the main board；Screw off the screw on the main board by screw driver；Take out the main board from the front housing module by plastic rods and separate the headset jack rubber from the main board。

g. Disassemble the sub board

Remove the main FPC from the sub board；Remove the coaxial cable connector from the sub board；Screw off the screw on the sub board by screw driver；Take out the sub board from the front housing module by plastic rods and separate the USB rubber from the sub board。

h. Disassemble the battery

Turn on the heating platform, and regulate temperature controller, set the target temperature to 80 degrees；After the heating platform is heated to 80 degrees, put the battery upwards on the heating platform horizontally；After the phone is heated for 3 min , take the phone with the heat insulation gloves；Immediately pull the battery out with a sucker。

i. Disassemble the battery

To separate the main FPC from the front housing module；To separate the Coaxial cable from the front housing module

j. Disassemble the TP module

Take out the front camera rubber from the housing module；Remove the speaker from the front housing module；Remove the motor from the front housing module；Remove the MIC1 rubber and MIC2 rubber from the front housing module

k. Disassemble the TP module

Turn on the heating platform, and regulate temperature controller, set the target temperature to 80 degrees；After the heating platform is heated to 80 degrees, put the phone screen downwards on the heating platform horizontally；After the TP module is heated for 5 min , take the phone with the heat insulation gloves；Put the phone screen downwards on the TP module disassembly fixture immediately；Open the disassemble equipment，To separate the TP module；Take the TP module out of the fixture， To separate the TP module with a plectrum。

l. Disassemble the TP module

To separate the p-sensor rubber and p-sensor  FPC from the front housing；Remove side key Mylar from the front housing；Screw off screw and remove the steel piece from the front housing；To separate the side key FPC from the front housing；Remove the receiver mesh conductive cloth and separate the receiver mesh from the TP module；Remove TP graphite film from the TP module。

The main board layout is introduced

2.1 The mainboard positive figure

2.2 Main board on the back of the figure

2.3 Motherboard, figure a PDF document

2.4 The main board schematic PDF document

3.Failure analysis flow chart

3.1 Don't start up

3.2 Touch screen failure

3.3 charge fault

3.4 The receiver malfunction

3.5 The horn fault

3.6 LCD display failure

3.7 The key fault

3.8 Mobile communication failures

3.9 MIC fault

更换CPU

3.10 Camera malfunction

3.10.1 The main camera malfunction

3.10.2 Before the camera malfunction

3.11 Vibration fault

3.12 SIM card not to identify

3.13 SD card can't identify

3.14 FM fault

3.15 Bluetooth/WLAN fault

3.16 Fingerprint fault