# OSLO-M-ROW Trouble Shooting Guide

# V1.0

Contents

OSLO-M-ROW Trouble Shooting Guide	1

V1.0	1

1. MB/KB illustration	3

2. Power on failure	4

3. Screen is not bright	5

4. Screen display abnormality	6

5. Touch screen failure	6

6. Optical/distance sensor error	7

7. G-sensor and gyroscope failure	8

8．Key failure	9

9. No charging	10

10. SIM card or SD card identification failure	11

11. Main camera function failure	12

12. Slave camera function failure	13

13. Flashlight failure	14

14. Motor error	15

15. GSM network failure	17

16. WCDMA and LTE network failure	18

17. WIFI/Bluetooth/FM	19

18. GPS failure	20

19.Self-calibration (SELF_CAL) and XO calibration failure：	21

20.Headset identification failure/headset MIC errors	22

21. MIC silent/low volume	23

22. Speaker silent	25

23. IF station FASTBOOT_FLASH_01	26

24. IF station WAIT_FOR_ALL_INTERFACE_ARRIVAL_01	26

# 1. MB/KB illustration

Sensor Hub

# 2. Power on failure

Solution:

- Check battery connector for soldering errors and its body for oxidation or damage;

- Check each power supply in the power sequence for short circuits with GND or for any voltage errors;

- Check power on circuit for any errors;

- For power on current around 20MA, check crystal for soldering errors;

- For power on current errors above 1A, check VBAT circuit for short circuits to ground, troubleshoot ground diode at individual locations;

- For power-on current errors above 100MA, check for hot point on MB and troubleshoot;

- Check soldering of each main chip with X-ray, including CPU/Flash/PMIC/TR IC;

- Add-solder or re-solder main chip, validate soldering conditions;

- Replace CPU/Flash/PMIC/LDO/TR IC;

# 3. Screen is not bright

Solution:

- Check LCD connector for dirt, PIN continuous tin, cold solder joint, PIN collapse, etc. Check the connector for proper connection and re-install it;

- LCD backlight powered by U1302, check whether output voltage falls is about 17V;

- Otherwise, check whether R1315、R1316、R1317、D1301、Q1301 is conductive, with soldering error or is damaged;

- Try re-placing D1301;

- Try re-placing D1302;

# 4. Screen display abnormality

Solution:

- Re-install LCD display screen or replace it before powering on the phone and check again;

- Check LCD connector(J2601) for dirt, PIN continuous tin, cold solder joint, PIN collapse, etc.;

- Check LCD filter (U2616、U2617、U2618、U2619、U2620) for soldering errors or damage;

- Check LCD peripheral components. Compare them with those on normal board to check for missing parts. Test supply voltages and check whether it is normal;

- Check CPU for over-gluing, in case an X-ray equipment is available, check the IC solder ball with it;

- Add-solder or re-solder CPU or replace it;

# 5. Touch screen failure

Solution:

- Check TP connector on FPC for proper connection, Re-install CTP or replace it before powering on and testing it again;

- Check whether LCD connector (J2601) is dirty, PIN continuous tin, cold solder joint, PIN collapse;

- Measure TP VDD supply voltage and check whether it is normal. Measure earth impedance of RST, INT, I2C information and check whether it is normal;

- Check the CPU soldering status with an X-ray, add-solder or re-solder it before testing it again;

- Replace CPU;

# 6. Optical/distance sensor error

Solution:

- Check optical chip surface for dirt or blockage; Check whether the plastic cap is well mounted;

- Check optical/distance sensor for damage or soldering error;

- Measure whether the VREG_L22_3P3 voltage on both ends of R2522 is normal;

- Measure whether the VREG_L6_1P8 voltage on both ends of R2501 is normal;

- Re-place optical and distance sensor FPC;

- Add-solder or re-solder CPU or re-place it;

# 7. G-sensor and gyroscope failure

Solution:

- Check the sensor module circuit components for soldering errors or damage;

- Check whether it's functionless or function error;

- Troubleshoot peripheral and channel devices

- Re-solder or re-place the sensor U2504;

- Re-solder or re-place the CPU;

# 8．Key failure

Solution:

- Check whether the dome falls off or is collapsed, or test the side key FPC after replacing it;

- Check Volume UP/Volume down/Power On voltage signal validity;

- Check components on key circuit for damage or soldering errors;

- Re-solder or replace the CPU or PMIC;

# 9. No charging

Solution:

- Check USB connector on the motherboard for dirt, damage, cold solder joint or other phenomena;

- Check battery connector J3202 for dirt, damage, bubble soldering;

- Re-place battery before testing again;

- Check whether the voltage on L1201 channel is normal

- Check the power management chip U1201 for soldering errors;

# 10. SIM card or SD card identification failure

Solution:

- Visually check SIM/SD tray for soldering offset, float or body damage;

- Remove J2903, check whether the DETECT dome positioned correctly (should be connected with metal casing).

- Check SIM/SD tray on the motherboard for float, dome plate contact, spring plate deformation and other errors;

- Measure the SIM power supply voltage VREG_L14_UIM1, Check whether the UIM1, VREG_L15_UIM2 is powering up;

- Check whether the ground resistance of SIM/SD signal wire is normal; Otherwise, disconnect serial connected resistor and test the CPU side.

- Re-solder or re-place U401 CPU;

# 11. Main camera function failure

Solution:

- Check camera installation for any abnormalities, re-install it or replace it before testing again;

- Check the connector of the main camera J2712 for damage or soldering errors, re-solder or re-place the connector;

- Check voltage of VREG_L17_2P8 /VREG_L2_1P2 /VREG_L6_1P8;

- Check U2713/U2714/U2716/ U2718/ U2719 EMI filter for damage or soldering error;

- Add-solder or re-solder PMIC or re-place it;

- Add-solder or re-solder CPU or re-place it;

# 12. Slave camera function failure

Solution:

- Check camera installation for any abnormalities, re-install it or replace it before testing again;

- Check the slave camera J2701 connector for damage or soldering errors, re-solder or re-place the connector;

- Check CAM_AVDD / FCAM_DVDD /VREG_L6_1P8 for voltage errors, if so, check PMIC for output errors or peripheral circuits for short circuits or broken circuits;

- Check U2703/U2706/U2707/U2720/U2721 EMI filter for damage or soldering errors;

- Check whether the LDO chip(U2704) or its soldering is normal;

- Add-solder or re-solder CPU;

# 13. Flashlight failure

Solution:

- Solder or re-place U1201;

# 14. Motor error

Solution:

- Check whether the R101, R102 is normal;

- Check whether MOTOR102，MOTOR103 is normal;

- Replace the motor;

- Re-solder power management chip U1202 or replace it;

# 15. GSM network failure

Solution:

- Check PCBA antenna dome soldering status, contact with phone casing antenna or damaged body;

- Check the connection of RF connector and circuit components, Inductors, resistors for soldering errors after collision;

- Check front RF switch FEM3301 and relevant components for soldering errors and body damage;

- Re-calibrate motherboard RF indicators. If it fails, troubleshoot and repair it for failed bands;

- Re-solder or replace FEM3301 RF transceiver chip;

- Re-solder or re-place U4300;

- Re-solder or re-place U4001;

- Re-solder or re-place U401;

# 16. WCDMA and LTE network failure

Solution:

- Check whether main and branch set test seat is falsely soldered;

- Calibrate phone MB, pinpoint cause of error, determine receiving or transmitting errors according to bands with errors, troubleshoot relevant circuit components;

- Add-solder or replace FEM3301, PA3401, 4300, duplex DU, Quad QP, Filter FL, RF Transceiver U4001;

- According to RF indicators, check components in the circuit at the corresponding band for soldering errors or body damage;

- Troubleshoot relevant band of diversity filter module according to source drawing;

- Re-solder or re-place U401 CPU;

# 17. WIFI/Bluetooth/FM

Solution:

- Check MB’s WIFI/BT antenna channel components or their soldering for damage;

- Check the RF test stand for false solder joints;

- Check headset channel connection, FM antenna circuit in headset;

- Check filter FL4401, DI4401 for soldering offset or body damage;

- Add-solder or re-solder WLAN/BT/FM 3-in-1 chip or re-place it before testing again;

- Add-solder or re-solder CPU U401, or re-place it before testing it again;

# 18. GPS failure

Solution:

- Check GPS channel components for soldering or body errors;

- Check the RF test stand for false solder joints;

- Check DI4401 for soldering offset or body damage;

- Check filter FL4501, FL4502 for soldering or body errors;

- Check LNA chip LNA4501 for soldering or body errors;

- Add-solder or re-solder U4001 chip or re-place it before testing again;

- Add-solder or re-solder CPU U401, or re-place it before testing it again;

# 19.Self-calibration (SELF_CAL) and XO calibration failure：

Solution:

- Check whether 5 mipi communication are OK, if so, identify errors for individual analysis or take the following steps:

- Check whether U4001 is of false solder joints, continuous tin or poor quality; Re-solder or re-place U4001;

- Check whether FEM3301 is of false solder joints, continuous tin or poor quality; Re-solder or re-place FEM3301;

- Check whether the U4001 is of false solder joints, continuous tin or poor quality; Re-solder or re-place U4001;

- Check whether the G1401 is of false solder joints, continuous tin or poor quality; Re-solder or re-place G1401;

- Check whether the U1401 is of false solder joints, continuous tin or poor quality; Re-solder or re-place U1401;

# 20.Headset identification failure/headset MIC errors

Solution:

- Re-test headset function after replacing headset;

- Check headset jack for damage or soldering for skews or over height, re-solder or replace headset jack;

- Check receiver circuit components for damage or soldering for errors, repair before testing it again;

- Re-solder or re-place codec U1801;

# 21. MIC silent/low volume

Solution:

- Visually check primary and secondary MIC relevant components for soldering offset or body damage;

- Re-place primary and secondary MIC before testing it again. Pay attention to high temperature during re-placement as it may damage it.

- Add-solder or re-solder U1201, or re-place it before testing again;

# 22. Speaker silent

Solution:

- Visual check relevant components for damage or soldering error;

- Check the MB dome and speaker FPC for poor soldering including skew and continuous tin;

- Re-solder or re-place U2304 or smart PA U2311.

# 23. IF station FASTBOOT_FLASH_01

Solution:

- Check whether the earth voltage of VREF_LPDDR3 is normal (0.6V or so).

- X-ray U401 to check for continuous tin

- Check the resistances around CPU and check R404

# 24. IF station WAIT_FOR_ALL_INTERFACE_ARRIVAL_01

Solution:

- Power on the USB and check whether the current of the motherboard is too large. If more than 100mA, find the power supply that is short-circuited

- Measure the resistance of USB_HS_DP, USB_HS_DM to check whether the earth impedance is abnormal;

- Check U101 for soldering errors;